# Fourteenth International Conference on Input-Output Techniques
October 10-15, 2002, Montréal, Canada

# Physical Input-Output Tables of Finland 1995 – solutions to some basic methodological problems

Ilmo Mäenpää

Thule Institute at the University of Oulu, Finland

ilmo.maenpaa@oulu.fi

# Abstract

In the compilation of physical input-output tables, the law of the conservation of matter is a basic criterion that enables double-entry bookkeeping of physical flows. As the law is valid only for the mass of materials, its application presupposes the measurement of physical flows in kilograms.

The law of the conservation of matter is fundamental: the mass of the outputs of any process has to be the same as the mass of the entered inputs. In practical applications, however, problems arise because processes often involve unobservable material flows. Examples of such unobservable flows are water vapour in drying processes, oxygen intake in fuel combustion, and carbon dioxide expiration in the metabolism of animals and humans.

The focus of this paper is to introduce exact calculation methods for making the unobservable flows visible in three problematic areas: 1) moisture changes of materials in refining processes, 2) balances of fuel combustion, and 3) metabolic balances of animals and humans.

This paper is based on the ongoing work on the compilation of physical input-output tables for Finland in 1995 – FINPIOT95 - at the Thule Institute and the Statistics Finland.

# 1	Introduction

Material flow accounting, MFA, is generally defined as the compilation and analysis of statistical data about the material inflows and outflows of an economy. The subject of the accounting is generally a national economy but the subject may be a region or an economic sector, too. The accounts may be oriented to the aggregative description of the inflows and outflows or it may be focused on a sector of an economy or on a region. The accounting may be aimed at describing the total mass inflows and outflows or it may be intended to a more detailed process analysis of different substances even inside the economy.

Material balance can be spoken about when the accounting consists of an all-inclusive total description of the material inflows and outflows of the system and the materials stored in the system.

Material flow accounting has recently reached official international status as The European Commission published a manual of the material flow accounting (European Commission 2001) – or MFA Guide for short - where the adoption of the material flow accounting at the national statistical offices of the EU is recommended. The unified conceptual and measurement framework has been developed, too.

Another current work aiming at the adoption of internationally unified material flow accounting methods is the revision of the System of Integrated Economic and Environmental Accounting (SEEA) of the United Nations (United Nations 1993). The draft worked out, SEEA 2000, has been placed on the Internet for public commenting. In Chapter III of the revised version a much wider and more systematic attention has been paid to the material flow accounting than in the version of 1993.

The most comprehensive and systematic material flow accounts are reached with the physical input-output tables (PIOT). In the physical input-output tables the flows of the materials through the economy are described in detail: the intake of natural resources by industries, their flows in the processing and refining processes, their final use and further their outflow into nature.

So far the physical input-output tables have been compiled only in two countries, in Germany for the year 1990 (Stahmer, Kuhn & Braun 1998) and in Denmark for the same year (Pedersen 1999). The compilation of physical input-output tables has been started in Finland, too, (Mäenpää & Muukkonen 2001) in FINPIOT project, with which this work is associated. Also the SEEA 2000 includes a rather throughout description of the material flow accounting in accordance with the input-output framework and the revised national accounting (Commission of the European Communities,... 1993, European Commission 1996).

The basis of the material flow accounting is the law of the conservation of matter: matter (mass or energy) cannot be created or destroyed by any physical process, but only change from one form to another. According to the theory of relativity the mass can be changed into energy and vice versa, but in the human techno-sphere only to such a small extent that in general it is below our measurement precision (even in the nuclear power plants the mass loss of the nuclear fuel is about 0.003 per cent). Hence with the practical measurement precision the conservation of mass and energy can be simplified as the principle of the conservation of mass.

The conservation of matter refers to the mass of materials, which in the field of gravitation appears as the weight of the materials. Thus in the material flow accounting all material flows have to be measured in mass units, in kilograms or in (metric) tons.

The principle of the conservation of mass gives the following basic equation of the material balance:

The mass input of the system = the mass output of the system

+ the changes in the mass of the material stocks inside the system.

In the description of the material flows of an economy this principle is demonstrated in Figure 1.

Figure 1. The economy-wide material balance (adopted from: European Commission 2001, 16)

By the law of the conservation of matter some total balance equations are valid in accordance with Figure 1, too. First, in the input side

Domestic extraction + direct imports

= material throughput of the economy + net additions to stocks.

Second, in the output side

The mass of materials ended up in domestic nature

= material throughput of the economy – exports.

The economy-wide material balance can be compiled directly with aggregate data. However, for analysing material flows inside the economy a more disaggregate input-output approach is needed. The Physical Input-Output Tables (PIOT) have been compiled for Germany (Stahmer et. al. 1998) and Denmark (Pedersen 1999).

This paper describes methods developed in the compilation of the Finnish PIOT of the year 1995 – or FINPIOT95. The preliminary work and its early results have been more extensively described in the paper Mäenpää & Muukkonen (2001).

At the compilation level the product and industry classifications of the FINPIOT95 are the same as in the monetary input-output tables of Finland (Statistics Finland 1999). Thus there are 190 industries and 1300 products, from which about 1200 are material goods. The PIOT are comprised of product x industry supply table, product x industry use table and industry x industry symmetric table associated with flows from and into nature.

A useful summary table of the input-output flows by industries is presented in Annex 1. The columns are constructed so that when the inner flows of the economy are cancelled, the figures below the bottom line give the external economy-wide flows of Figure 1.

The table of Annex 1 contains the flows of both the energy carriers and “ordinary” materials. Annex 2 presents the material flows of fuel combustion separately and Annex 3 the flows associated with the “ordinary” materials. The tables are still preliminary since some sectors need more accurate estimations.

In the FINPIOT95 project much work has been addressed to develop systematic and exact methods for the computer software of the compilation which are then easy and quick to adapt for the next update of the PIOT. Below we focus on methods in three problem areas where unobservable flows are central in balancing the material flows, namely:

- changes in water content of products

- material balances of fuel combustion

- material flows of biological metabolism.

# 2	Changes in water content of products

The water content of products entering the economy in primary production is often high: 25 – 90 % by plants, 45 % by wood and about 10 % by minerals. In basic industries, where those raw materials are processed to intermediate and final products, the moisture of the material flows is generally lowered. Most often the drying is realised by evaporation in which case the water leaving the product flow remains unobserved.

Some products like beverages and paints include added water.

To keep the mass balances of material flows in order, moisture changes or additions of water and evaporation have to be included in balances.

For managing the moisture changes in the compilation of the PIOT a relatively simple method can be developed, however.

Let mi be the moisture rate of product i, uij the use of product i by the industry j and sij the supply of product i by industry j. Then

wj  = Σi misij - Σi miuij  = Σi mi(sij - uij ),    and  |wj|  = water input, if wj > 0

=  water vapour output, if wj < 0.

The equation can be presented more comprehensively in matrix form as follows. Let m be the vector of moisture rates of products, S the product x industry supply matrix and U the product x industry use matrix. Then

w = m’S – m’U = m’(S-U), 	 where   |wj|  = water input, if wj > 0

=  water vapour output, if wj < 0.

The FINPIOT95 include about 1200 material products. The moisture rates are estimated for 275 products at the beginning of the processing chains. For the refined products the water content is generally so low that the moisture changes have no significance. Working with product by product moisture vector means, however, that new data about significant moisture contents can be easily included in the compilation system.

Annex 3 shows that industries where the share of released water vapour is high are especially forest industries, manufacture of food products and manufacture of non-metallic minerals. Added water can be found in manufacture of beverages, in manufacture of chemicals and in manufacture of non-metallic minerals. The water shares in inputs and outputs are presented separately for food and forest industries in Table 1.

Table 1. Shares of the added water in inputs and of water vapour in outputs of selected industries

|  |  | Input |  |  | Output |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Water | Products | Total | Products | Water | Total |
| 4 | Manufacture of food products | 0.7 | 99.3 | 100.0 | 79.0 | 21.0 | 100.0 |
| 5 | Manuf.of beverages & tobacco | 56.1 | 43.9 | 100.0 | 99.7 | 0.3 | 100.0 |
| 7 | Manuf of wood & wood products | 0.0 | 100.0 | 100.0 | 79.7 | 20.3 | 100.0 |
| 8 | Manuf of pulp, paper, paper prod | 0.0 | 100.0 | 100.0 | 72.2 | 27.8 | 100.0 |

# 3	Combustion of fuels

The combustion of fuels combines the combustible substances of fuels and the oxygen of the air and thus transforms them into gases. The heat born by the combustion evaporates the water contained by the fuel, too. The incombustible substances of the fuel are left as ash.

The material flows of the combustion could be accounted in principle from the substance composition of the fuels. The substance composition of some fuels is expressed in Table 2. To ease the presentation only data for the 11 most important fuels are shown. The FINPIOT95 include 33 fuel types altogether. The data is mainly from Alakangas (2000) and Kurki-Suonio (1981).

In the conditions of complete combustion the material flows of combustion could be accounted exactly from the data of substance compositions: from the elements of the fuel the carbon C nitrogen N and sulphur S are oxidized into CO2, NO2 (mainly) and SO2. The hydrogen H is oxidized into water H2O and is combined with the vapour of the water originally contained by the fuel. The external oxygen required is obtained by subtracting the oxygen contained by the fuel from the oxygen used to oxidize the other elements of the fuel.

Figure 2. Material flows of combustion

Table 2. Energy content and substance composition of the most important fuels

|  | Energy | Substance composition, % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MJ/kg | Water | Ash | C | H | O | N | S | Total |
| Liquefied Petroleum Gas | 45.6 | 0.00 | 0.00 | 0.82 | 0.18 | 0.00 | 0.00 | 0.00 | 1.00 |
| Motor Gasoline | 43.1 | 0.00 | 0.00 | 0.87 | 0.13 | 0.00 | 0.00 | 0.00 | 1.00 |
| Kerosene Type Jet Fuel | 43.1 | 0.00 | 0.00 | 0.86 | 0.14 | 0.00 | 0.00 | 0.00 | 1.00 |
| Light Fuel Oil | 42.3 | 0.00 | 0.00 | 0.86 | 0.13 | 0.00 | 0.00 | 0.01 | 1.00 |
| Heavy Fuel Oil | 41.1 | 0.00 | 0.00 | 0.88 | 0.10 | 0.00 | 0.00 | 0.01 | 1.00 |
| Hard Coal | 25.2 | 0.10 | 0.13 | 0.66 | 0.03 | 0.07 | 0.01 | 0.01 | 1.00 |
| Natural gas | 49.3 | 0.00 | 0.00 | 0.74 | 0.24 | 0.01 | 0.01 | 0.00 | 1.00 |
| Peat | 12.3 | 0.40 | 0.03 | 0.31 | 0.03 | 0.21 | 0.01 | 0.01 | 1.00 |
| Fuel Wood | 11.6 | 0.23 | 0.01 | 0.38 | 0.05 | 0.33 | 0.01 | 0.00 | 1.00 |
| Black Liqour | 11.7 | 0.25 | 0.02 | 0.36 | 0.04 | 0.31 | 0.00 | 0.03 | 1.00 |
| Nuclear Fuel | 3 025 210.1 | 0.00 | 1.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.00 |

In practice, however, the combustion is incomplete and emissions are reduced with end-of-pipe measures. In incomplete combustion part of the combustible substances remains in the ash, part of the carbon is oxidized only into carbon monoxide CO or is released as methane CH4. In high temperatures part of nitrogen contained in the combustion air is oxidized, too. Thus e.g. the exhaust fumes of cars have high nitrous oxide emissions although the transport fuels do not contain nitrogen.

The “empirical” emission coefficients of fuels for the year 1995 obtained from the accounts of emission statistics of Finland are shown in Table 3. The emission coefficients are expressed generally in relation to the energy unit of fuels (kg/MJ), but in Table 3 they are converted to the relation of mass unit of fuels (kg/kg). By most of the fuels the share of the carbon dioxides is over 99 % of the total emissions. Thus in rough accounts the other emissions could be neglected, too. However, including them in accounts is useful for maintaining the exact connection with the emission statistics.

With the empirical emission coefficients the external oxygen and oxidized nitrogen of the air can be obtained by the equation:

Oxygen and nitrogen from air = (Ash + Water vapour + Emissions to air) – Mass of fuel.

When accounting per mass unit of fuel the ash is obtained directly from the Ash column of Table 2, the water is obtained by adding to the Water column of the table the water formed by oxidizing hydrogen, or multiplying the H column by nine. The emissions to air are obtained from Table 3.

Table 3.  Emission coefficients of the most important fuels according to emissions statistics, kg/kg

|  | CO2 | NOx | SO2 | CH4 | CO | N2O | Total | CO2/Total, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Liquefied Petroleum Gas | 2.776 | 0.0049 | 0.0007 | 0.0002 | 0.001 | 0.0001 | 2.783 | 99.7 |
| Motor Gasoline | 3.123 | 0.0386 | 0.0002 | 0.0010 | 0.146 | 0.0003 | 3.309 | 94.4 |
| Kerosene Type Jet Fuel | 3.052 | 0.0093 | 0.0004 | 0.0041 | 0.019 | 0.0015 | 3.086 | 98.9 |
| Light Fuel Oil | 3.101 | 0.0184 | 0.0013 | 0.0003 | 0.008 | 0.0004 | 3.129 | 99.1 |
| Heavy Fuel Oil | 3.149 | 0.0080 | 0.0176 | 0.0003 | 0.001 | 0.0001 | 3.177 | 99.1 |
| Hard Coal | 2.337 | 0.0045 | 0.0050 | 0.0001 | 0.000 | 0.0001 | 2.347 | 99.6 |
| Natural Gas | 2.754 | 0.0052 | 0.0000 | 0.0002 | 0.001 | 0.0001 | 2.761 | 99.8 |
| Peat | 1.291 | 0.0021 | 0.0021 | 0.0001 | 0.001 | 0.0002 | 1.296 | 99.6 |
| Fuel Wood | 1.260 | 0.0007 | 0.0000 | 0.0035 | 0.013 | 0.0000 | 1.278 | 98.6 |
| Black Liqour | 1.273 | 0.0008 | 0.0002 | 0.0000 | 0.002 | 0.0000 | 1.275 | 99.8 |
| Nuclear Fuel | 0.000 | 0.0000 | 0.0000 | 0.0000 | 0.000 | 0.0000 | 0.000 | - |

Table 4 shows the material flow balances of combustion obtained. In FINPIOT95 the material balances of combustion are obtained by multiplying the fuel use of industries .by the figures of the table.

Table 4. Material balances of fuel combustion per mass unit of fuels (kg/kg)

|  | Input |  |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fuel | O2,N2 | Total | Ash | Vapour | Emissions | Total |
| Liquefied Petroleum Gas | 1 | 3.37 | 4.37 | 0.00 | 1.59 | 2.78 | 4.37 |
| Motor Gasoline | 1 | 3.47 | 4.47 | 0.00 | 1.17 | 3.31 | 4.47 |
| Kerosene Type Jet Fuel | 1 | 3.39 | 4.39 | 0.00 | 1.30 | 3.09 | 4.39 |
| Light Fuel Oil | 1 | 3.31 | 4.31 | 0.00 | 1.18 | 3.13 | 4.31 |
| Heavy Fuel Oil | 1 | 3.09 | 4.09 | 0.00 | 0.91 | 3.18 | 4.09 |
| Hard Coal | 1 | 1.89 | 2.89 | 0.13 | 0.41 | 2.35 | 2.89 |
| Natural gas | 1 | 3.96 | 4.96 | 0.00 | 2.20 | 2.76 | 4.96 |
| Peat | 1 | 1.03 | 2.03 | 0.03 | 0.71 | 1.30 | 2.03 |
| Fuel Wood | 1 | 0.93 | 1.93 | 0.01 | 0.64 | 1.28 | 1.93 |
| Black Liqour | 1 | 0.86 | 1.86 | 0.02 | 0.57 | 1.28 | 1.86 |
| Nuclear Fuel | 1 | 0.00 | 1.00 | 1.00 | 0.00 | 0.00 | 1.00 |

# 4	Metabolic material balances of domestic animals

The material content of living organisms can be divided into following nutrients:

- carbohydrates

- proteins

- fats

- alcohol

- minerals

- water.

The first four are called energy nutrients because their combustion is the basis of the energy production of the living organisms. Of the energy nutrients alcohol belongs to human nutrition only. Carbohydrates are often divided further into easily digested carbohydrates and indigestible fibres.

The nutrient composition of the plant and animal products contained in food chain has been extensively cleared up by nutrition research. In Table 5 some examples of these are presented.

Table 5. Nutrient composition of plant and animal products, some examples

|  | Nutrient composition, % on average |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proteins | Fats | Carbo- | Fibres | Minerals | Water | Total |
|  |  |  | hydrates |  |  |  |  |
| Plants |  |  |  |  |  |  |  |
| Potato | 2.1 | 0.0 | 18.0 | 0.7 | 1.2 | 78.0 | 100.0 |
| Wheat | 11.9 | 1.9 | 67.7 | 2.8 | 1.7 | 14.0 | 100.0 |
| Pasture grass | 3.4 | 0.8 | 8.8 | 4.2 | 1.8 | 81.0 | 100.0 |
| Tomato | 1.0 | 0.3 | 3.5 | 0.7 | 0.5 | 94.0 | 100.0 |
| Garden berries | 0.5 | 0.6 | 7.5 | 1.9 | 0.5 | 89.1 | 100.0 |
| Animals |  |  |  |  |  |  |  |
| Beef cattle | 18.0 | 10.0 | 0.0 | 0.0 | 4.0 | 68.0 | 100.0 |
| Fattening pig | 12.0 | 36.0 | 0.0 | 0.0 | 2.6 | 49.4 | 100.0 |
| Milk | 3.3 | 4.3 | 4.3 | 0.0 | 0.7 | 87.4 | 100.0 |
| Egg | 13.0 | 10.5 | 0.0 | 0.0 | 10.5 | 66.0 | 100.0 |
| Human body | 18.0 | 18.0 | 0.0 | 0.0 | 4.3 | 59.7 | 100.0 |

Energy nutrients are composed mainly of four elements: carbon (C), hydrogen (H), oxygen (O) and nitrogen (N). Nitrogen is included only in proteins. Each energy nutrient has a characteristic molecular structure and even though especially proteins and fats are comprised of a variety of molecules, the elementary composition of each of them is relatively similar. Table 6 presents the average relative elementary composition of energy nutrients.

Table 6. Average elementary composition of energy nutrients, % in mass units

| Elements | Carbohydrates | Fats | Proteins | Alcohol |
| --- | --- | --- | --- | --- |
| C | 44 | 77 | 54 | 52 |
| H | 6 | 12 | 7 | 13 |
| O | 50 | 11 | 23 | 35 |
| N |  |  | 16 |  |
| Total | 100 | 100 | 100 | 100 |

Using the elementary composition the material balances of combustion of energy nutrients can be derived as presented in Table 7. Note that the biological combustion of proteins is incomplete in the meaning that the end product of the oxidation of nitrogen in proteins is not nitrous oxide but by mammals urea, (NH2)2CO, and by birds uric acid, C5H4N4O3.

Table 7.  Summary of the material balances of nutrients combustion kg/kg

|  | INPUT |  |  | OUTPUT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nutrient | O2 | Total | CO2 | H2O | Urea/ | Total |
|  |  |  |  |  |  | uric acid |  |
| Carbohydrates | 1.00 | 1.15 | 2.15 | 1.61 | 0.54 |  | 2.15 |
| Fats | 1.00 | 2.90 | 3.90 | 2.82 | 1.08 |  | 3.90 |
| Proteins, mammals | 1.00 | 1.49 | 2.49 | 1.73 | 0.42 | 0.34 | 2.49 |
| Proteins, birds | 1.00 | 1.36 | 2.36 | 1.35 | 0.53 | 0.48 | 2.36 |
| Ethyl alcohol | 1.00 | 2.08 | 3.08 | 1.91 | 1.17 |  | 3.08 |

The metabolic balances are derived only for the main domestic animal types in Finland: dairy cows, beef cattle, sows, fattening pigs, hens and broilers. Since the life time of many animal types is less than a year, the observation unit is defined as animal place year, which contain the reproduction requirements of the animals, too.

The amounts of different feeds used by the animal places are presented in Table 8.

Table 8. The amounts of feeds per animal place, kg/year

|  | Dairy | Beef | Sow | Fattening | Hen | Broiler | Fur |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | cow | cattle |  | pig |  |  | animal |
| barley (1-03) | 685 | 326 | 554 | 373 | 10 | 0.2 | 0.0 |
| oats (1-08) | 1 385 | 0 | 699 | 0 | 10 | 0.2 | 0.0 |
| hay (19-04) | 2 729 | 0 | 0 | 0 | 0 | 0.0 | 0.0 |
| silage (18-30) | 10 734 | 6 834 | 0 | 0 | 0 | 0.0 | 0.0 |
| pasture grass (16-03) | 5 126 | 0 | 0 | 0 | 0 | 0.0 | 0.0 |
| Own feed, total | 20 658 | 7 160 | 1 253 | 373 | 20 | 0.3 | 0.0 |
| protein concentrate (5-34) | 864 | 258 | 263 | 0 | 0 | 0.0 | 0.0 |
| mineral feed (5-04) | 88 | 26 | 0 | 0 | 0 | 0.0 | 0.0 |
| growth feed (5-35) | 0 | 47 | 0 | 300 | 0 | 16.8 | 0.0 |
| drink feed (8-30) | 0 | 21 | 760 | 0 | 0 | 0.0 | 0.0 |
| egg laying extract (5-26) | 0 | 0 | 0 | 0 | 17 | 0.0 | 0.0 |
| poultry limestone | 0 | 0 | 0 | 0 | 3 | 0.0 | 0.0 |
| feed for fur animals | 0 | 0 | 0 | 0 | 0 | 0 | 84.5 |
| Concentrated feed, total | 952 | 352 | 1 023 | 300 | 20 | 16.8 | 84.5 |

In feed research the nutrient composition and digestibility properties of feeds are investigated circumstantially. Table 9 shows the nutrient compositions of the feeds. As in the nutrition studies in general, carbohydrates are further divided into digestible carbohydrates and indigestible fibres.

Table 9. Nutrient composition of feeds, %

|  | Proteins | Fats | Carbo- | Fibres | Minerals | Water | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | hydrates |  |  |  |  |
| barley (1-03) | 10.8 | 1.9 | 65.9 | 4.9 | 2.5 | 14.0 | 100.0 |
| oats (1-08) | 11.2 | 5.2 | 55.9 | 10.7 | 3.1 | 14.0 | 100.0 |
| hay (19-04) | 8.3 | 1.7 | 39.8 | 29.1 | 4.2 | 17.0 | 100.0 |
| silage (18-30) | 3.7 | 1.1 | 9.5 | 6.6 | 2.1 | 77.0 | 100.0 |
| pasture grass (16-03) | 3.4 | 0.8 | 8.8 | 4.2 | 1.8 | 81.0 | 100.0 |
| protein concentrate (5-34) | 43.9 | 7.2 | 27.1 | 5.2 | 5.7 | 11.0 | 100.0 |
| mineral feed (5-04) | 41.4 | 0.9 | 27.0 | 14.4 | 6.3 | 10.0 | 100.0 |
| growth feed (5-35) | 45.8 | 3.0 | 28.2 | 5.1 | 5.9 | 12.0 | 100.0 |
| drinking feed (8-30) | 7.0 | 0.9 | 13.1 | 0.1 | 0.9 | 78.0 | 100.0 |
| egg laying extract (5-26) | 34.4 | 3.9 | 32.1 | 11.5 | 7.1 | 11.0 | 100.0 |
| poultry limestone | 0.0 | 0.0 | 0.0 | 0.0 | 90.0 | 10.0 | 100.0 |
| feed for fur animals | 8.2 | 9.0 | 3.8 | 0.0 | 9.0 | 70.0 | 100.0 |

Table 10 presents the digestibility per cents of the feeds. The digestibility expresses how much the animal can ultimately exploit the nutrients and how much is directly excreted as faeces. The digestibility of feeds differs between animal types because based on the differences of the digestive system the digestibility of feeds differs between animals.

Table 10. Digestibility of feeds by animal types, %

|  | Proteins | Fats | Carbo- | Fibres |
| --- | --- | --- | --- | --- |
|  |  |  | hydrates |  |
| Ruminant animals |  |  |  |  |
| barley (1-03) | 72 | 75 | 89 | 30 |
| oats (1-08) | 77 | 90 | 76 | 30 |
| hay (19-04) | 56 | 50 | 61 | 60 |
| silage (18-30) | 74 | 73 | 74 | 75 |
| pasture grass (16-03) | 74 | 60 | 80 | 80 |
| protein concentrate (5-34) | 90 | 83 | 91 | 70 |
| mineral feed (5-04) | 86 | 34 | 60 | 25 |
| growth feed (5-35) | 90 | 74 | 91 | 70 |
| drinking feed (8-30) | 87 | 72 | 95 | 50 |
| Pigs |  |  |  |  |
| barley (1-03) | 71 | 55 | 89 | 15 |
| oats (1-08) | 74 | 82 | 77 | 15 |
| protein concentrate (5-34) | 88 | 78 | 91 | 60 |
| growth feed (5-35) | 88 | 63 | 91 | 60 |
| drinking feed (8-30) | 93 | 89 | 96 | 10 |
| Poultry |  |  |  |  |
| barley (1-03) | 64 | 60 | 77 | 0 |
| oats (1-08) | 68 | 84 | 63 | 0 |
| growth feed (5-35) | 87 | 50 | 36 | 0 |
| egg laying extract (5-26) | 80 | 50 | 30 | 0 |
| Fur animals |  |  |  |  |
| feed for fur animals | 85 | 90 | 78 | 0 |

In the following we use dairy cow as an example, how the metabolic balances of animals can be derived. The compilation table for the metabolic balance of a dairy cow is presented in Table 11.

The intake rows include first eaten feed by nutrient content and the amount of drinking water. The oxygen intake by respiration is determined on the use side by the oxygen consumed in energy production.

Table 11. The metabolic balance of dairy cow, kg/year

|  | Proteins | Fats | Carbo- | Fibres | Minerals | Water | Oxygen | Carbon | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | hydrates |  |  |  |  | dioxide | kg/year |
| Intake |  |  |  |  |  |  |  |  |  |
| Feed | 1 305 | 312 | 3 610 | 1 685 | 477 | 11 534 |  |  | 18 922 |
| Water |  |  |  |  |  | 32 850 |  |  | 32 850 |
| Respiration |  |  |  |  |  |  | 4 963 |  | 4 963 |
| Total | 1 305 | 312 | 3 610 | 1 685 | 477 | 44 384 | 4 963 | 0 | 56 735 |
| Use |  |  |  |  |  |  |  |  |  |
| Faeces | 308 | 81 | 917 | 597 | 238 | 8 565 |  |  | 10 706 |
| Urine | 268 |  |  |  | 190 | 7 171 |  |  | 7 628 |
| Methane |  |  | 94 | 38 |  |  |  |  | 132 |
| Milk | 184 | 252 | 266 | 0 | 40 | 5 438 |  |  | 6 180 |
| Calf | 10 | 2 | 0 | 0 | 2 | 37 |  |  | 50 |
| Heifer growth | 25 | 16 | 0 | 0 | 7 | 85 |  |  | 133 |
| - 1/4 calf | -2 | 0 | 0 | 0 | -1 | -9 |  |  | -13 |
| Energy production | 513 | -38 | 2 333 | 1 049 |  | -3 151 | 4 963 | -5 670 | 0 |
| Respiration, exhalation |  |  |  |  |  | 26 249 |  | 5 670 | 31 919 |
| Total | 1 305 | 312 | 3 610 | 1 685 | 477 | 44 384 | 4 963 | 0 | 56 735 |

On the use side the indigestible parts of energy nutrients and minerals leave the body in faeces. Urea formed from the protein used in energy production and minerals from renewing the tissue compounds leave in urine. The water content of faeces and urine is determined by parameters of dry-matter shares of them. The methane yield is determined by digestion of carbohydrates and fibres. Part of the digested nutrients is tied up directly by the animal products, by milk, calves and growth of the heifer. As each fourth calf is used for reproduction and transformed into a heifer, it has to be subtracted for the sake of accounting consistency.

The energy nutrients left after the tying up of animal products are available for energy production. The amounts of energy nutrients used for the combustion are first in the row of energy production – here the share of urea is subtracted from protein and thus the protein is a so-called non-urea protein. The amount of fats available for energy production is negative. The reason for this is that into animal products, in this case into milk, more fats are tied up than available in digested food. In the animal organisms this deficit is compensated by transforming carbohydrates into fats. In the row of energy production the negative fats calculate correctly the effect of this transformation to the elementary composition of the combusted nutrients and to their energy values. The water and carbon dioxide yielded and the oxygen consumed by combustion of the energy nutrients are calculated by means of the material balances of the combustion of nutrients. The so-called metabolic water formed by combustion joins the water equilibrium and the carbon dioxide leaves in respiration. The water leaving in respiration and exhalation is determined by the water equilibrium

The results of the metabolic balances can also be presented as a reduced external material balance as in Table 12. In Balance 1 the inputs and outputs are classified as products, and in Balance 2 they are analysed into dry-matter, water and gas content of the products, The total sum of the external balances is a little higher than of the metabolic balance, because the ¼ of a calf is in the external balance removed to the input side.

Table 12. External material balances of dairy cow, kg/year

Balance 1   		       		                      Balance 2

| INPUT | 56 748 | OUTPUT | 56 748 |  | INPUT | 56 748 | OUTPUT | 56 748 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Feed | 18 922 | Milk | 6 180 |  | Dry-matter | 7 391 | Dry-matter | 3 401 |
| 1/4 calf | 13 | Calf | 50 |  | Water | 44 394 | Water | 21 295 |
| Water | 32 850 | Heifer | 133 |  | Gas | 4 963 | Gas | 32 051 |
| Oxygen | 4 963 | Faeces | 10 706 |  |  |  |  |  |
|  |  | Urine | 7 628 |  |  |  |  |  |
|  |  | Water vapour | 26 249 |  |  |  |  |  |
|  |  | Carbon dioxide | 5 670 |  |  |  |  |  |
|  |  | Methane | 132 |  |  |  |  |  |

Balance 2 shows the considerable changes of the forms of material existence caused by the metabolism. The dry-matter and liquid water content of outputs are nearly half of the content of inputs. Instead over six times more gases evaporate into the air than gases are taken in.

Introducing metabolic balances for each type of domestic animals and raising the accounts to the total animal places in Finland, we get the total material flows of the animal husbandry as presented in Table 13. The table has been supplemented by the additional material flows of manure management: straw for solid and rinsing water for liquid manure in the input side and ammonia and some methane emissions in the output side.

Table 13. Total material balances of the animal husbandry in Finland 1995, 1000 tonnes

|  | Dairy | Beef | Sow | Fattening | Hen | Broiler | Fur | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | cow | cattle |  | pig |  |  | animal |  |
| Animal places, 1000 units | 399 | 340 | 95 | 670 | 4300 | 5100 | 3749 |  |
| Input | 29 227 | 9 781 | 1 490 | 4 131 | 584 | 657 | 457 | 46 328 |
| Forage plants | 8 243 | 2 434 | 119 | 250 | 87 | 2 | 0 | 11 134 |
| Concentrated feed | 380 | 120 | 97 | 201 | 86 | 86 | 317 | 1 286 |
| Straw | 330 | 65 | 35 | 0 | 0 | 0 | 0 | 430 |
| Water (drinking & rinsing) | 18 294 | 6 664 | 1 153 | 3 541 | 356 | 558 | 46 | 30 613 |
| Oxygen | 1 980 | 498 | 86 | 140 | 56 | 11 | 94 | 2 864 |
| Output | 29 227 | 9 781 | 1 490 | 4 131 | 584 | 657 | 457 | 46 327 |
| Milk | 2 466 | 0 | 0 | 0 | 0 | 0 | 0 | 2 466 |
| Calves | 15 | -15 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beef cattle | 53 | 140 | 0 | 0 | 0 | 0 | 0 | 193 |
| Pigs | 0 | 0 | 45 | -44 | 0 | 0 | 0 | 1 |
| Pork | 0 | 0 | 22 | 217 | 0 | 0 | 0 | 239 |
| Eggs | 0 | 0 | 0 | 0 | 77 | -2 | 0 | 75 |
| Chicken | 0 | 0 | 0 | 0 | -5 | 0 | 0 | -5 |
| Poultry meat | 0 | 0 | 0 | 0 | 5 | 61 | 0 | 66 |
| Fur skins | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| Slaughtering waste | 0 | 0 | 0 | 0 | 0 | 0 | 16 | 16 |
| Manure | 13 877 | 3 694 | 1 083 | 2 686 | 268 | 141 | 283 | 22 031 |
| Ammonia | 20 | 6 | 1 | 5 | 1 | 1 | 1 | 37 |
| Methane | 61 | 13 | 2 | 5 | 0 | 0 | 0 | 80 |
| Carbon dioxide | 2 262 | 551 | 105 | 149 | 63 | -1 | 76 | 3 205 |
| Water vapour | 10 473 | 5 391 | 232 | 1 115 | 174 | 457 | 79 | 17 921 |

The total external material balance of animal husbandry is presented in Table 14 and in Figure 3. The figure shows the huge transformation of the form of existence of materials brought by the animal husbandry.

Table 14. Aggregated external material balance of Finnish animal husbandry in 1995, million tonnes

| INPUT | 45.3 | OUTPUT | 45.3 |
| --- | --- | --- | --- |
| Dry-matter | 5.1 | Dry-matter | 2.8 |
| Water | 37.4 | Water | 21.7 |
| Gas | 2.8 | Gas | 20.8 |

Figure 3. Material flows of animal husbandry in Finland 1995, million tonnes

# 5	Human metabolic balances

As a basis for the metabolic material balances of Finnish population the foodstuffs consumption of Finnish households are first estimated. The household consumption inquiry of the year1998 by Statistics Finland included additional inquiry of the physical quantities of acquired foodstuffs and numbers of different kinds of food and beverage portions in restaurants and canteens. The restaurant and canteen portions have been converted back to their raw foodstuff contents and both the direct foodstuff acquisition of households and the foodstuff acquisition of restaurants and canteens are changes into levels of the year 1995 by means of the fixed price consumption expenditure time series of the national accounts. The food acquisition in the year 1995 per capita is presented in Table 15. To introduce the ultimate food intake, a general food loss of 3 % is assumed and additionally losses due to inedible parts of foodstuffs (skins, bones etc.) are estimated.

Table 15. Food acquisition, losses and intake of households in 1995, kg per capita and intake as energy, MJ per capita

|  | Acquisition, kg |  |  | Loss | Intake | Energy |
| --- | --- | --- | --- | --- | --- | --- |
|  | Home | Restaurant | Total | % | kg | MJ |
| 1 Flour, other grain products | 36 | 5.7 | 41.7 | 3 | 40.4 | 504 |
| 2 Bread and biscuits | 36.9 | 2.6 | 39.5 | 3 | 38.3 | 391 |
| 3 Beef | 2.6 | 1.5 | 4.1 | 12.5 | 3.6 | 21 |
| 4 Pork | 7.3 | 1.4 | 8.6 | 15 | 7.4 | 58 |
| 5 Poultry | 3.2 | 0.1 | 3.3 | 30.2 | 2.3 | 10 |
| 6 Other meat | 1.4 | 0 | 1.4 | 18.2 | 1.1 | 5 |
| 7 Sausages and other meat products | 33.3 | 1.7 | 35 | 3.2 | 33.9 | 291 |
| 8 Fish | 10.9 | 2.3 | 13.2 | 15.3 | 11.2 | 57 |
| 9 Milk | 130.3 | 11.1 | 141.3 | 3 | 137.1 | 250 |
| 10 Dairy products | 50.5 | 5 | 55.6 | 3 | 53.9 | 326 |
| 11 Eggs | 8.1 | 0.7 | 8.8 | 13.7 | 7.6 | 44 |
| 12 Oils and fats | 11.1 | 3.6 | 14.7 | 3 | 14.3 | 414 |
| 13 Fruits and berries | 60 | 0.1 | 60.1 | 19.5 | 48.4 | 112 |
| 14 Potato | 51.1 | 7 | 58.1 | 16.6 | 48.4 | 123 |
| 15 Potato products | 4 | 0 | 4 | 3 | 3.8 | 52 |
| 16 Other plants | 39.4 | 8.7 | 48.2 | 8.9 | 43.9 | 52 |
| 17 Sugar, jam, honey, confectionery | 25.3 | 0.5 | 25.8 | 3 | 25 | 358 |
| 18 Other foodstuff | 1.8 | 0.2 | 2.1 | 3 | 2 | 5 |
| 19 Coffee, tea and cocoa | 7.8 | 0.2 | 8 | 76.6 | 1.9 | 6 |
| 20 Other non-alcoholic beverages | 61.1 | 18.9 | 80.1 | 3 | 77.6 | 102 |
| 21 Alcoholic beverages | 86.4 | 30.2 | 118.2 | 3 | 114.6 | 268 |
| Total | 668.5 | 101.5 | 771.6 | 7.1 | 716.8 | 3449 |
| Per person / day |  |  | 2.1 |  | 2 | 9.5 |

For foodstuffs comprehensive nutrient content databank, FINELI, is available in Finland at internet (FINELI 2001), by means of which the nutrient composition of the foodstuffs has been estimated. The nutrient composition of the foodstuffs is presented in Table 16.

In human nutrition studies the following digestibility rates of energy nutrients are generally assumed:

- Proteins		  92 %

- Fats			  95 %

- Alcohol	  	100 %

- Carbohydrates		  97 %

- Fibres			    0 %.

The compilation table for the material balance of an average person, as presented in Table 17, can be constructed similarly as for dairy cow above.

The metabolic balance has been changed into external material balances in Table 6.9 and in Table 6.10 they have been raised further to the level of the total Finnish population.

Table 16. Nutrient composition and physiological fuel values of foodstuffs

|  | Nutrient content, % |  |  |  |  |  |  |  | Energy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Carbo- | Fats | Proteins | Alcohol | Fibres | Minerals | Water | Total | MJ/kg |
|  | hydrates |  |  |  |  |  |  |  |  |
| 1 Flour, other grain products | 47.2 | 8.1 | 9.2 | 0.0 | 4.4 | 1.7 | 29.4 | 100.0 | 12.5 |
| 2 Bread and biscuits | 42.7 | 4.4 | 8.6 | 0.0 | 6.1 | 2.3 | 36.0 | 100.0 | 10.2 |
| 3 Beef | 0.0 | 8.4 | 19.3 | 0.0 | 0.0 | 0.9 | 71.4 | 100.0 | 5.8 |
| 4 Pork | 0.1 | 16.5 | 17.0 | 0.0 | 0.0 | 1.7 | 64.7 | 100.0 | 7.9 |
| 5 Poultry | 0.2 | 6.4 | 21.3 | 0.0 | 0.0 | 0.9 | 71.2 | 100.0 | 4.3 |
| 6 Other meat | 0.0 | 5.5 | 20.8 | 0.0 | 0.0 | 1.2 | 72.5 | 100.0 | 4.7 |
| 7 Sausages and other meat products | 4.5 | 14.3 | 14.7 | 0.0 | 0.2 | 2.5 | 63.7 | 100.0 | 8.6 |
| 8 Fish | 2.5 | 7.2 | 16.3 | 0.0 | 0.1 | 3.0 | 70.9 | 100.0 | 5.1 |
| 9 Milk | 4.9 | 1.3 | 3.1 | 0.0 | 0.0 | 0.7 | 90.0 | 100.0 | 1.8 |
| 10 Dairy products | 5.8 | 9.9 | 8.2 | 0.0 | 0.1 | 1.4 | 74.7 | 100.0 | 6.1 |
| 11 Eggs | 0.3 | 11.8 | 12.1 | 0.0 | 0.0 | 1.1 | 74.6 | 100.0 | 5.8 |
| 12 Oils and fats | 0.2 | 76.7 | 0.6 | 0.0 | 0.0 | 1.1 | 21.3 | 100.0 | 29.0 |
| 13 Fruits and berries | 14.0 | 0.9 | 0.7 | 0.0 | 2.3 | 0.5 | 81.5 | 100.0 | 2.3 |
| 14 Potato | 15.7 | 0.1 | 1.7 | 0.0 | 1.2 | 1.1 | 80.3 | 100.0 | 2.5 |
| 15 Potato products | 46.7 | 13.1 | 4.4 | 0.0 | 2.0 | 1.7 | 32.1 | 100.0 | 13.4 |
| 16 Other plants | 4.9 | 0.6 | 1.3 | 0.0 | 1.8 | 0.9 | 90.5 | 100.0 | 1.2 |
| 17 Sugar, jam, honey, confectionery | 71.1 | 5.5 | 2.2 | 0.0 | 0.5 | 0.7 | 20.0 | 100.0 | 14.3 |
| 18 Other foodstuff | 7.6 | 3.0 | 1.7 | 0.0 | 0.2 | 52.2 | 35.3 | 100.0 | 2.7 |
| 19 Coffee, tea and cocoa | 42.4 | 5.7 | 21.5 | 0.0 | 3.7 | 18.1 | 8.7 | 100.0 | 3.1 |
| 20 Other non-alcoholic beverages | 7.7 | 0.0 | 0.1 | 0.0 | 0.0 | 0.2 | 92.0 | 100.0 | 1.3 |
| 21 Alcoholic beverages | 3.8 | 0.0 | 0.3 | 5.6 | 0.0 | 0.1 | 90.1 | 100.0 | 2.3 |
| On average | 13.2 | 4.8 | 4.1 | 0.9 | 1.0 | 1.1 | 74.9 | 100.0 | 4.8 |

Table 17.  Human metabolic balance in 1995, g/person/day

|  | Proteins | Fats | Alcohol | Carbo- | Fibres | Minerals | Water | Oxygen | Carbon | Total | Energy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | hydrates |  |  |  |  | dioxide | g/day | kJ/day |
| Intake |  |  |  |  |  |  |  |  |  |  |  |
| Meals | 93.1 | 78.6 | 0.0 | 228.4 | 18.9 | 20.3 | 992.5 |  |  | 1 432 | 9 653 |
| Beverages | 0.3 | 2.4 | 17.5 | 30.6 | 0.2 | 1.7 | 479.3 |  |  | 532 | 1 157 |
| Drinking water |  |  |  |  |  |  | 754.3 |  |  | 754 |  |
| Respiration |  |  |  |  |  |  |  | 674.3 |  | 674 |  |
| Total | 93.5 | 81.0 | 17.5 | 259.0 | 19.1 | 22.0 | 2 226.0 | 674.3 | 0.0 | 3 392 | 10 810 |
| Use |  |  |  |  |  |  |  |  |  |  |  |
| Faeces | 7.5 | 4.0 | 0.0 | 7.8 | 19.1 | 11.0 | 100.0 |  |  | 149 | 809 |
| Urine | 29.4 |  |  |  |  | 10.9 | 1 500.0 |  |  | 1 540 | 311 |
| Methane |  |  |  | 2.5 |  |  |  |  |  | 3 | 139 |
| Growth of body mass | 0.4 | 0.4 |  |  |  | 0.1 | 1.3 |  |  | 2 | 25 |
| Energy production | 56.2 | 76.5 | 17.5 | 248.7 |  |  | -274.0 | 674.3 | -799.3 | 0 | 9 525 |
| Respiration, perspiration |  |  |  |  |  |  | 898.7 |  | 799.3 | 1 698 |  |
| Total | 93.5 | 81.0 | 17.5 | 259.0 | 19.1 | 22.0 | 2 226.0 | 674.3 | 0.0 | 3 392 | 10 810 |

.

Table 18. External material balances of humans in 1995, g/person/day

Balance 1   		       		             Balance 2

| INPUT | 3 392 | OUTPUT | 3 392 |  | INPUT | 3 392 | OUTPUT | 3 392 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meals | 1 432 | Body mass | 2 |  | Dry-matter | 492 | Dry-matter | 91 |
| Beverages | 532 | Faeces | 149 |  | Water | 2 226 | Water | 1 601 |
| Water | 754 | Urine | 1 540 |  | Gas | 674 | Gas | 1 700 |
| Oxygen | 674 | Water vapour | 899 |  |  |  |  |  |
|  |  | Carbon dioxide | 799 |  |  |  |  |  |
|  |  | Methane | 3 |  |  |  |  |  |

Table 19. External metabolic material balances of Finnish population in 1995, 1000 tonnes

Balance 1   		       		             Balance 2

| INPUT | 6 336 | OUTPUT | 6 336 |  | INPUT | 6 336 | OUTPUT | 6 336 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Food | 2 674 | Body mass | 4 |  | Dry-matter | 919 | Dry-matter | 169 |
| Drinks | 994 | Faeces | 279 |  | Water | 4 158 | Water | 2 991 |
| Water | 1 409 | Urine | 2 877 |  | Gas | 1 259 | Gas | 3 176 |
| Oxygen | 1 259 | Water vapour | 1 678 |  |  |  |  |  |
|  |  | Carbon dioxide | 1 493 |  |  |  |  |  |
|  |  | Methane | 5 |  |  |  |  |  |

Figure 5. Metabolic material balance of Finnish population in 1995, million tonnes

# 5	Conclusions

The law of the conservation of matter gives a firm foundation for the material flow accounting provided that material flows are measured uniformly in mass units, kilograms or tonnes.

This paper shows that there are many cases where unobservable flows are either absorbed or released by the registered observable flows and thus cause remarkable statistical discrepancies in balance accounts if we confine ourselves only to the apparent statistical data.

Three such cases are handled: moisture changes of products especially at the beginning of the refining process chains, fuel combustion and biological metabolism of domestic animals and humans.

However, exact methods can be developed to link the unobservable flows to the observed data. These methods are based on the basic physical and chemical properties of matter and thus they are firmer than e.g. relations based on the behaviour of economic institutions.

The methods can be included in the computer software routines of the material flow accounting and thus they will ease the compilation of material balances in future.

# References

Alakangas, Eija 2000. Suomessa käytettävien polttoaineiden ominaisuuksia. Valtion teknillinen tutkimuskeskus, VTT Tiedotteita 2045. Espoo

Kurki-Suonio, I. 1981. Polttoaineet ja palaminen. Tekniikan käsikirja osa 2. Yleiset perusteet. Jyväskylä.

European commission 2001. Economy-wide material flow accounts and derived indicators, a methodological guide. Eurostat Theme 2, Economy and finance, Luxemburg.

Mäenpää, I. & Muukkonen, J. 2001. Physical Input-Output in Finland: Methods, Preliminary Results and Tasks Ahead. Paper presented at Workshop on Economic growth, material flows and environmental pressure, 25th  – 27th April 2001, Stockholm, Sweden. Also in: http://www.thule.oulu.fi/assets/pdf/Stoc.pdf

Mäenpää, I. & Vanhala, P. 2002. Biological metabolism and the balances of food flows in Finland 1995. Draft 3 July 2002

SEEA 2000. 2002 Chapter III, Physical flow accounts. http://ww2.statcan.ca/citygrp/london/publicrev/ pubrev.htm, 22 Feb. 2002

Pedersen, O.G. 1999. Physical Input-Output Tables for Denmark; Products and Materials 1990; Air Emissions 1990-92. Danmarks Statistik.

Stahmer, C., Kuhn, M. & Braun, N. 1998. Physical Input-Output Tables for Germany, 1990, German Federal Statistical Office, prepared for DG XI and Eurostat, 2/1998/B/1

Statistics Finland 1999. Input-Output 1995. SVT Economy 1999:10, Helsinki.

FINELI 2001. Elintarvikkeiden koostumustietopankki, Kansanterveyslaitos, ravitsemusyksikkö, http://www.ktl.fi/fineli/ , Updated 15.6.2001

ANNEX 1

ANNEX 2

ANNEX 3

The material of this and following chapter are based on the manuscript Mäenpää & Vanhala (2002).