# Paper for the 16-th International Input-Output Conference, Istanbul Turkey, 2-6 July 2007

# Calculation and analysis of a hybrid energy input-output table for Germany within the Environmental-Economic Accounts (EEA)

by

Helmut Mayer

## 1. Introduction

The energetic input-output (I/O) analysis is an important method in the German Environmental Economic Accounts (EEA) in the analysis of energy, especially on the sources for energy demand and the effects of final use and production on energy consumption. Both the direct and the indirect consumption of energy are analysed. Direct energy is analysed by calculating adequate indices for energy consumption, e.g. consumption of primary energy of branches and households or intensities on energy consumption for branches. By means of the decomposition analysis the determining factors for energy demand – the level of demand, the composition of demand and the intensity of energy use – are analysed. This analysis is done regularly both for the energy consumption of households and for the energy use of the branches in the production process.

The energy used in the production of products (goods and services) – the indirect energy consumption of final use – is thoroughly analysed by using the energetic I/O-analysis. Therefore indirect energy consumption related to the final demand of products – produced domestically or abroad - and caused by the final use categories is calculated regularly on the basis of the monetary input-output tables (MIOT) provided by the input-output accounts of the Federal Statistical Office (FSO) and with the help of energy-flow tables of the EEA. The energy-flow tables are showing the consumption of energy for more than 70 branches and for households in a detailed breakdown to thirty energy carriers.

The following analysis and calculations are made in the EEA with the energetic I/O analysis:

- Analysis of the effects of increasing international integration (globalization) with respect to energy consumption and air emissions within the economy and abroad.

- Comparison of total cumulated energy consumption (direct and indirect consumption of energy) with domestic energy consumption (of primary energy).

- Calculation of import shares on total cumulated (primary) energy.

- Calculation of energy consumption according to final use categories (final consumption, gross capital formation, exports).

- Calculation of energy intensity for final demand categories in a breakdown to commodities.

- Determination of indirect energy related to the production of imports and exports.

- Calculation of indirect energy needed for the production of imported goods for final use and for intermediate consumption.

Those calculations have been made with an I/O-model using the Leontief inverse matrix on the basis of the monetary IOT (MIOT). This model is relatively demanding with regard to some conditions: it implies homogeneity of the rows (commodities) and columns (homogeneous production units) of the IOT. This requirement is difficult to fulfil. Improvements can be made by disaggregation of the IOT and/or by substituting monetary flows through (homogeneous) physical flows for certain inputs – like energy.

This paper shows how the energetic I/O-analysis has been improved in Germany on one hand by disaggregation of important energy producers and users (homogeneous branches) and on the other hand by integrating the physical energy flows into the IOT and constructing a hybrid (energy) IOT (HIOT).

The HIOT integrates the figures on the physical energy flows into the IOT (cf. figure 1). For the calculation of the Leontief-inverse both are used the physical and the monetary entries. The input coefficients therefore are of different type: for the energy rows either TJ/TJ or “mixed” TJ/EUR, for the other inputs they are of a mixed type EUR/TJ or “simple” EUR/EUR.

Figure 1: Hybrid energy Input-Output table

The Leontief inverse of the mixed model give more reliable results with regard to the energy content of final demand because they better reflect the “true” energy requirements of the supplying industries. On the basis of monetary models (MIOT) energy requirements of a certain demand are systematically underestimated if the supplying industries purchases energy at a price lower than the average. This is e.g. true for most of the basic industries which purchase electricity at a much lower price than service industries e.g. or households.

Comparisons of the results of the monetary model with the results from the mixed model showed significant differences, especially for the demand of products with a high content of “basic industries” – like chemicals, ferrous and non-ferrous metals.

In the following first the methods of calculating the energy flow tables are presented (chapter 2). Later in this chapter it is presented how the IOT is further disaggregated in the area of energy producers and important energy users like the chemical industry. In chapter 3 some results from the energetic I/O analysis done for 1995, 2000 and 2004 are presented.

## 2. Construction of the energy input-output tables

In the EEA energy flow tables are calculated regularly. Energy consumption is calculated for 71 branches (homogene

ous production units) for private households and for exports (including bunkering) and comprises a breakdown of about 30 energy carriers according to the classification of energy carriers in the national energy balances.  The breakdown of branches is the same as in the (monetary) IOT. For the construction of the energy IOT further disaggregation is applied for the energy branches and for selected energy-intensive branches (hier Verweis auf ausführliche Gliederung mit CPA im Anhang).

The calculation of the energy flow tables is closely linked to the figures in the energy balances: the data in the sub-balances for the various sectors (energy producing and transforming sectors, non-energetic use, manufacturing, traffic, households and agriculture, construction, trade and service industries) are the key figures for the calculation of the detailed energy consumption of branches:

Figure 2: Energy balance and energy flow tables

In the monetary IOT for Germany five energy branches are specified. This breakdown corresponds to the requirements of EUROSTAT for the annual transmission of SUT (supply and use tables) and IOT (every five years) from the member states to EUROSTAT. This breakdown of energy branches is not sufficient to get accurate results in the energetic I/O analysis. To get more homogeneous energy branches a further breakdown to 9 energy branches was carried out (cf. table 1).

Table 1: Disaggregation of energy branches in the IOT:

| CPA | Energy branches in the M-IOT | No. | CPA | Branches in the energy-IOT |
| --- | --- | --- | --- | --- |
| 10 | Mining of coal and lignite; extraction of peat | 1 | 10.1 | Hard coal and hard coal products |
|  |  | 2 | 10.2 | Lignite (brown coal) and lignite products, peat |
| 11 | Extraction of crude petroleum and natural gas; service activities | 3 | 11 | Extraction of crude petroleum and natural gas; service act. |
| 23 | Manufacture of coke, refined petroleum products and nuclear fuel | 4 | 23.1 | Coke |
|  |  | 5 | 23.2 | Refined petroleum products |
|  |  | 6 | 23.3 | Nuclear fuel |
| 40.1/3 | Production and distribution of electricity and heat | 7 | 40.1 | Electricity |
| 40.2 | Gas | 8 | 40.2 | Gas |
|  |  | 9 | 40.3 | Long-distance heat |

The new breakdown has a much better correspondence to the national energy balance which specifies 13 energy producing sectors. Among them five different types of electricity power plants are listed. Within this analysis it was not feasible to apply such a detailed breakdown of the electricity sector. However this could be a future task and would possibly allow further improvements. Using roughly the breakdown of the energy balance sheets, this enabled to utilise directly the figures for the transformation inputs and the consumption of the energy sectors. The non-energy inputs had to be estimated on the basis of commodity-flow tables from the input-output accounts and the structural business statistics.

Besides an adequate disaggregation of energy sectors, also a sufficient detail of energy intensive branches is necessary to get reliable results on indirect energy used for the production of goods. The MIOT already comprehends a disaggregation of important energy users:

Table 2: Energy-intensive homogeneous production units

| CPA | Classification of branches used in the MIOT for Germany |
| --- | --- |
| 21.1 | Pulp, paper and paper products |
| 25.1 | Rubber products |
| 25.2 | Plastic products |
| 27.1-3 | Basic iron, steel and tubes and semi-finished products made from basic iron, steel and tubes |
| 27.4 | Basic precious and non-ferrous metals and semi-finished products made from basic precious and non-ferrous metals |
| 27.5 | Casting of metals |
| 60.1 | Transport via railways |
| 60.2-3 | Other land transport, transport via pipelines |

Within the frame of the calculation of the energy IOT one further significant breakdown was made – the disaggregation of the chemical industry. The work of another disaggregation – of the non-ferrous industry - is in progress.

For the chemical industry the basic chemicals industry (CPA 24.1) was separated from the chemical industry (without pharmaceuticals, which are already shown as an own branch). The chemical industry is the most important energy consumer in production and especially for the production of basic chemicals a high specific amount of energy is needed. In 2004 total chemical industry had a consumption of primary energy of 1350 PJ. That is 13.1 % of total energy consumption of all branches. The basic chemicals industry alone had an energy consumption of 1269 PJ of which 69% (872 PJ) are used as raw materials for the production of basic chemicals (non-energy use). The basic chemicals show an energy content far above average. Therefore for the calculation of indirect energy a separate presentation of the basic chemical industry and a depiction of the links between the basic chemicals and the branches which further process basic chemicals is essential and improves the quality of the calculations.

Figure 3: Disaggregation of the chemical industry (NACE 24):

The disaggregation of the chemical industry in the energy IOT is planned in two steps: in a first step the sub-branch “manufacture of basic chemicals” (CPA 24.1) is shown as an own column and row in the IOT. This step has been already finalised and has led to substantial improvements of the results. In a second step the other groups of division 24 will be separated and integrated into the IOT also. This step needs further research especially with regard to the determination of the relationships between the basic chemical industry and the other sub-sectors of the chemical industry, but also with regard to their specific cost structures.

For the calculations of the inputs and outputs of the basic chemicals various sources were used:

- Use of basic chemicals for intermediate consumption and for final use (row of the IOT): figures were taken from the detailed commodity-flow table showing in great detail the use of basic chemicals both for output from domestic production and for imports.

- Energy inputs: the inputs were calculated in physical units as well as in monetary units. The figures for the physical energy inputs (in Terajoule) were taken from the energy balance sheets as well as from the energy statistics. In the energy balance sheets energy consumption within the basic chemical industry is shown separately as non-energy use and as final use of energy carriers. The physical energy inputs had to be valuated in order to get a total for intermediate consumption. This total was used as a check by comparing it with the figure for gross value added which has been calculated independently on the basis of the structural business statistics.

- Other (monetary) inputs: a wide range of inputs like raw materials could be selected from the figures for the whole chemical industry on the basis of the detailed entries in the commodity flow table. After determining and balancing raw materials and energy inputs with subtotals from the structural business statistics the remaining inputs – mostly services – were determined according the average input structure for the chemical industry in the MIOT.

Another important energy user in the manufacturing industries is the non-ferrous metals industry (CPA 27.4). This industry and especially the production of aluminium and aluminium products (CPA 27.42) require a great deal of energy. Therefore this industry will be also shown separately. For the calculation of energy used for the production of aluminium results from the life-cycle analysis (LCA) will be used.

Energy requirements of imported goods were compiled on the basis of the HIOT for Germany. It is assumed, that these goods would have been produced domestically, that means with the domestic technology. As a consequence not the actual energy content of the imports is compiled but the energy saved domestically by producing the goods abroad.

For imported goods for intermediate consumption the energy requirement matrix was compiled by subtraction. Two matrices were calculated. First, energy requirements for intermediate consumption of domestic origin were calculated by combining final demand from domestic output with the Leontief-Inverse for inputs from domestic production. Secondly, the same final use vector was combined with the Leontief-Inverse for total use. The first matrix was subtracted from the second one, showing then the energy requirements for imported products for intermediate consumption in a classification to branches (rows) and final use product categories (columns). This matrix provides additional information on the significance of imported goods for intermediate consumption and enables to integrate specific information on actual energy coefficients in production abroad.

## 3. Results

Calculations for indirect energy were made on the basis of the hybrid energy IOT for the years 1995/2000/2002/2003 and 2004. HIOT were calculated separately for total use of products (from domestic output and imports), for imported products and for products from domestic production. The calculations were done in a breakdown of 73 branches and for seven final use categories. For the final use categories matrices for the energy requirements were compiled in a classification to products of final use and branches which contribute to the production of these products. These matrices allow both an analysis of energy requirements according to final demand product categories and according to the producing branches engaged in the production of these goods.

In the following results for aggregated final use categories are presented. Table 3 shows the results of the calculation for direct, indirect and cumulated energy consumption for 1995, 2000 and 2004.

Total energy consumption (total supply) increased from 23.6 EJ to 24.8 EJ (+5.4 %). There was a slight drop in domestic production from 12.6 EJ in 1995 to 12.4 EJ in 2004, whereas direct imports of energy carriers rose by 13 % from 11.0 EJ to 12.4 EJ and contributed to half of total supply of direct energy. About 63 % (2004) of energy is consumed by the homogeneous branches, the rest by private households (24 %) and exports (11 %).

When looking at energy used for the production of goods and services – indirect energy – this leads to extremely interesting results: both the importance of imports and the significance of exports increases. Energy needed for the production of imported products got up by more than 20 % from 5.9 EJ to 7.2 EJ. Energy needed for the production of exported products jumped up by more than 40% from 6 EJ to 8.5 EJ. Therefore the production of exports absorbs now more energy (2004: 48.8 % of total primary energy) than the production of products for private consumption (2004: 32.4 %). Whereas energy for exports showed that boost, energy for private consumption products in absolute terms even dropped from 6.1 EJ to 5.7 EJ. Probably an increased energy efficiency in production and structural changes in the composition of consumption expenditures compensated for the volume increase in expenditures.

Table 3: Supply and use of energy 1995/2000/2004 (Petajoule)

Table 4 provides more information on the imports and exports and the energy needed for their production. Imports - in value terms – (row 1) increased more than domestic production (row 7) – by more than 70 % compared to an increase of 23 % for domestic output. Both the demand for imported goods for final use and for intermediate consumption contributed to that jump in total imports. But the exports increased even more: the exports incl. re-exports increased by more than 90 % from 1995 to 2004, without re-exports the rise is 86%.

The calculations for indirect energy show that while in 1995 the energy requirements for the production of imports exceeded the (net) exports by more than 650 PJ (row 19) the situation has totally changed since 2002. From there on the production of exports needs more energy than for imports. In 2004 the exports exceeded the imports by more than 160 PJ.  Almost half of the primary energy – exactly 46 % - consumed by domestic branches (row 17) is needed for the production of export products. In 1995 this percentage amounted to 36 %.

Table 4: Imports and exports of goods 1995/2000/2004 and consumption of energy for their production

The figures reflect an enhanced relationship of the domestic economy with the economy abroad both for the households and the domestic producers. The rise in imported goods for intermediate consumption indicates not only an overall expansion of exchanges with the foreign economies but a change in the international division of work. Especially the boost in the period 1995 to 2000 reflects the period of ‘outsourcing’. For Germany this period was characterised by an enormous capital investment in the Middle and East European countries and a substitution of own production activities through purchases from suppliers in the East.

4. Outlook

A main objective of the I/O analysis on energy in the EEA is to get reliable results about the pressure of the economic activities on the environment. This implies that the real consequences of the demand for imported goods should be analysed. For this analysis most often life-cycle analysis (LCA) and not I/O analysis is used, because I/O analysis fails to consider the often very special production processes in foreign economies. Nevertheless I/O analysis seems to be appropriate for such an analysis - especially for the analysis in the field of energy - also. Data for production processes abroad could be integrated in the calculations for imports, e.g. special energy coefficients for the production of specific goods. E.g. the production of electricity is the main source for the demand of primary energy. Concerning the imports for Germany about 29% of total energy is required by the “electricity branch” in the frame of production of imported goods. Data on the physical inputs and the input-output coefficients of power plants abroad are available and could be integrated in an hybrid energy IOT.

In the German EEA work is done to integrate data from the LCA for selected energy-intensive production processes abroad in the I/O analysis of energy and CO2-emissions. This work is part of an analysis on the consequences of globalisation on the environment. In that analysis especially the effects of a substitution of resource-intensive domestic processes by imports and the effects of the increased export activity on the use of environmental resources will be analysed in more detail. It is expected that the results will contribute to the question on the effects of globalisation on the environment.

References

Mayer, H. (1993): Entwicklung des Energiegehalts von Gütern 1978 bis 1988 (1993), in: Schnabl, H.: Ökointegrative Gesamtrechnung, Berlin, New York.

Mayer, H. (2006): Umweltökonomische Analysen im Bereich der Energie, Online publication of the EEA; (http://www.destatis.de/download/d/ugr/Umweltoekonomische_Analyse_der_Energie.pdf)

Mayer, H. (2006): Energieberechnungen in den Umweltökonomischen Gesamtrechnungen – Methoden, Umfang, Anwendungen; Online publication of the EEA, (http://www.destatis.de/download/d/ugr/energieberechnung_methodenbeschreibung.pdf).

Miller, R.E. and Blair, P. D. (1985) Input-Output Analysis: Foundation and Extensions (Englewood Cliffs, NJ: Prentice-Hall).

Statistisches Bundesamt (2006): Umweltnutzung und Wirtschaft, Bericht zu den Umweltökonomischen Gesamtrechnungen 2006 (report to the EEA 2006).

http://www.destatis.de/allg/d/veroe/proser4fumw2_d.htm

Statistisches Bundesamt (2004): Use of the Environment and the Economy, Tables on EEA 2006, Online available at DESTATIS: http://www.destatis.de/download/e/ugr/5_Energy.pdf

Statistisches Bundesamt (2007): Volkswirtschaftliche Gesamtrechnungen – Input-Output Rechnungen, subject-matter 18, series 2.

ANNEX:

Table 2 A: Classification of branches in the energy input-output for Germany

Federal Statistical Office of Germany, Division of Environmental-Economic Accounts (EEA).

Acknowledgment to Ms. Christine Flachmann who contributed to the calculations.

A research project with the Fraunhofer Institute Karlsruhe was initiated with the goal of determining the energy requirements of the chemicals sub-sectors and processes and to get figures for the linkages between the sub-sectors. Results are expected for the end of 2007.

Cf. Classification of branches in Annex, table 2.

An aggregated hybrid energy IOT for 2002 is annexed (table 2 of annex).  More detailed results will be presented later this year in the context of a press conference on the topic „environmental-economic consequences of globalisation” planned for November.